Table 6.2.11.1 Rural Residential Zone Code

| Performance Outcomes | Acceptable Outcomes |  | Proposed Solution Explanation of how the development addresses the Acceptable Outcome and/or Performance Outcome. |  |
| --- | --- | --- | --- | --- |
| For accepted, accepted subject to requirements and assessable development (code, code (fast tracked) and impact) |  |  |  |
| Building Height |  |  |  |
| PO1 A low-rise built form is maintained having regard to existing landscape character values. | AO1 Development has a maximum building height of 8.5 metres above ground level and two (2) storeys. |  |  |
| Accommodation Density |  |  |  |
| PO2 Accommodation density and residential density is complementary and subordinate to the semi-rural and natural landscape values of the area. | AO2.1 Residential density does not exceed one dwelling house per lot. AO2.2 Residential density does not exceed two dwellings per lot and development is for a secondary dwelling with a maximum GFA of 80m². |  |  |
| Setbacks |  |  |  |
| PO3 Building setbacks are appropriate having regard to: - the semi-rural character of the area; - overshadowing; - privacy and overlooking; and - the primary road frontage setbacks of adjoining premises. | AO3.1 Buildings and structures have a minimum setback of 15 metres to the primary road frontage. AO3.2 Buildings and structures have a minimum side and rear boundary clearance of 10 metres. |  |  |
| Site Cover |  |  |  |
| PO4 Development protects the semi-rural and natural landscape values of the area and is visually unobtrusive. | AO4.1 Site cover is a maximum of 20% of the total site area. Where in Precinct 1 – Rural Residential Precinct 4000 AO4.2 Domestic outbuildings ancillary to a dwelling have a maximum floor area of 150m2. Note—AO4.2 excludes balconies and verandahs where connected to a dwelling. AND AO4.3 Buildings and structures ancillary to a dwelling are restricted to a cumulative floor area of 200m2. Where in Precinct 2 – Rural Residential Precinct 8000 and Precinct 3 – Rural Residential Precinct 20000 AO4.4 Buildings and structures ancillary to a dwelling are restricted to a cumulative floor area of 200m2. Note—AO4.2 excludes balconies and verandahs where connected to a dwelling. |  |  |
| For assessable development (code, code (fast tracked) and impact) |  |  |  |
| Amenity Protection |  |  |  |
| PO5 Development must not detract from the amenity of the local area, having regard to: - noise; - hours of operation; - traffic; - lighting; - advertising devices; - visual amenity; - privacy; - odour; or - emissions. | AO5 No acceptable outcome. |  |  |
| PO6 Development must take into account and seek to ameliorate any existing negative environmental impacts, having regard to: - noise; - hours of operation; - traffic; - lighting; - advertising devices; - visual amenity; - privacy; - odour; or - emissions. | AO6 No acceptable outcome. |  |  |
| Water Quality Management |  |  |  |
| PO7 Development protects environmental values and facilitates the achievement of water quality objectives for Queensland waters. | AO7 No acceptable outcome. |  |  |
| PO8 Development achieves the stormwater management design objectives specified in Table 6.2.11.2 – Construction Phase - Stormwater Management Design Objectives. | AO8 Development achieves objectives as specified in Table 6.2.11.2 – Construction Phase - Stormwater Management Design Objectives. |  |  |
| PO9 Land for urban purposes is located in areas which avoid or minimise the disturbance to natural drainage, areas subject to erosion risk and groundwater. | AO9 No acceptable outcome. |  |  |
| PO10 Land for urban purpose is located, designed, constructed and managed to avoid impacts arising from altered stormwater quality or flow. | AO10 No acceptable outcome. |  |  |

Table 6.2.11.2—Construction Phase – Stormwater Management Design Objectives

| Issue |  | Design Objectives |
| --- | --- | --- |
| Drainage control | Temporary drainage works | - Design life and design storm for temporary drainage works: - Disturbed area open for <12 months - 1 in 2-year ARI event. - Disturbed area open for 12-24 months - 1 in 5-year ARI event. - Disturbed are open for >24 months - 1 in 10-year ARI event. - Design capacity excludes minimum 150mm freeboard. - Temporary culvert crossing - minimum 1 in 1-year SRI hydraulic capacity. |
| Erosion control | Erosion control measures | - Minimise exposure of disturbed soils at any time. - Divert water run-off from undisturbed areas around disturbed areas. - Determine the erosion risk rating using local rainfall erosivity, rainfall depth, soil-loss rate or other acceptable methods. - Implement erosion control methods corresponding to identified erosion risk rating. |
| Sediment control | Sediment control measures Design storm for sediment control basins Sediment basin dewatering | - Determine appropriate sediment control measures using: - potential soil loss rate, or - monthly erosivity, or - average monthly rainfall - Collect and drain stormwater from disturbed soils to sediment basin for design storm event: - design storm for sediment basin sizing is 80th% five-day event or similar - Site discharge during sediment basin dewatering: - TSS < 50 mg/L TSS, and - Turbidity not >10% receiving waters turbidity, and - pH 6.5–8.5 |
| Water quality | Litter and other waste, hydrocarbons and other contaminants | - Avoid wind-blown litter; remove gross pollutants. - Ensure there is no visible oil or grease sheen on released waters. - Dispose of waste containing contaminants at authorised facilities. |
| Waterway stability and flood flow management | Changes to the natural waterway hydraulics and hydrology | - For peak flow for the 1-year and 100-year ARI event, use constructed sediment basins to attenuate the discharge rate of stormwater from the site. |