Table 6.2.12.1 Community Facilities Zone Code

Proposed Solution

Explanation of how the development addresses the Acceptable Outcome and/or Performance Outcome.

| Performance Outcomes | Acceptable Outcomes |  | Proposed Solution Explanation of how the development addresses the Acceptable Outcome and/or Performance Outcome. |  |
| --- | --- | --- | --- | --- |
| For accepted, accepted subject to requirements and assessable development (code, code (fast tracked) and impact) |  |  |  |
| Building Height |  |  |  |
| PO1 A low-rise built form is maintained having regard to: - overshadowing and privacy of accommodation activities and land in a Residential zone category; - building character and appearance; and - the height of buildings on adjoining premises. | AO1 Development has a maximum building height of 9.5 metres above natural ground level and no more than two storeys. |  |  |
| Gross Floor Area |  |  |  |
| PO2 The scale and bulk of built form is complementary to existing development in the locality. | AO2 Development has a maximum gross floor area of 50% of the site area. |  |  |
| Site Cover |  |  |  |
| PO3 The site coverage of all buildings and structures does not result in a built form that is bulky and visually intrusive. | AO3 Site cover is a maximum of: - for a single storey building - 50% of the total site area; or - for a 2 or more storey building - 40% of the total site area. |  |  |
| Setbacks |  |  |  |
| PO4 Building setbacks are appropriate having regard to: - efficient use of the site; - overshadowing; - privacy and overlooking; - building character and appearance; and - the primary road frontage setbacks of adjoining premises. | AO4.1 Buildings and structures have a minimum setback of 6 metres to the primary road frontage. OR AO4.2 Buildings and structures have a road frontage setback equal to or greater than the setback of an existing building on the premises. AO4.3 Buildings and structures have a minimum side and rear boundary clearance of 2.5 metres. Where adjoining land in a Residential Zone category AO4.4 Buildings and structures have a minimum side and rear boundary clearance of 3 metres. |  |  |
| For assessable development (code, code (fast tracked) and impact) |  |  |  |
| Building Appearance |  |  |  |
| PO5 Development must be complementary to and integrate with the existing character and visual amenity of the area. | AO5 Building services and equipment are screened so as not to be visible from the road and other public areas or adjoining residences. |  |  |
| Landscaping |  |  |  |
| PO6 Landscaping is provided to contribute to the visual amenity of the premises and local area. | AO6.1 A minimum of one shade tree is provided for every six car parking spaces. AO6.2 A minimum planting space of 1.2m² is provided for every shade tree. AO6.3 A landscape buffer with a minimum width of 1 metre is provided to all vehicle movement and car parking areas adjacent to buildings and site boundaries. Where adjoining a Dwelling or a use in the Residential Zone category AO6.4 A 2 metre minimum landscape buffer is provided along the shared boundary. |  |  |
| Non-Discriminatory Access |  |  |  |
| PO7 Non-discriminatory access must be provided to the building from the road. | AO7 Changes of level between the road and the building must comply with Australian Standard AS1428—Design for access and mobility. |  |  |
| Amenity Protection |  |  |  |
| PO8 Development must not detract from the amenity of the local area, having regard to: - noise; - traffic; - lighting; - advertising devices; - visual amenity; - privacy; - odour; or - emissions. | AO8 No acceptable outcome. |  |  |
| PO9 Development must take into account and seek to ameliorate any existing negative environmental impacts, having regard to: - noise; - hours of operation; - traffic; - lighting; - advertising devices; - visual amenity; - privacy; - odour; or - emissions. | AO9 No acceptable outcome. |  |  |
| Where adjoining land in a Residential Zone category PO10 Development must not detract from the amenity of the local area having regard to operating hours. | Where adjoining land in a Residential Zone category AO10.1 Loading and unloading of goods is restricted to between the following hours: - 7.00am and 6.00pm Monday to Friday; - 8.00am and 5.00pm Saturdays. AO10.2 No unloading or loading occurs on Sundays and public holidays. |  |  |
| Water Quality Management |  |  |  |
| PO11 Development protects environmental values and facilitates the achievement of water quality objectives for Queensland waters. | AO11 No acceptable outcome. |  |  |
| PO12 Development achieves the stormwater management design objectives specified in Table 6.2.12.2 – Construction Phase - Stormwater Management Design Objectives. | AO12 Development achieves objectives as specified in Table 6.2.12.2 – Construction Phase - Stormwater Management Design Objectives. |  |  |
| PO13 Land for urban purposes is located in areas which avoid or minimise the disturbance to natural drainage, areas subject to erosion risk and groundwater. | AO13 No acceptable outcome. |  |  |
| PO14 Land for urban purpose is located, designed, constructed and managed to avoid impacts arising from altered stormwater quality or flow. | AO14 No acceptable outcome. |  |  |

Table 6.2.12.2—Construction Phase – Stormwater Management Design Objectives

| Issue |  | Design Objectives |
| --- | --- | --- |
| Drainage control | Temporary drainage works | - Design life and design storm for temporary drainage works: - Disturbed area open for <12 months - 1 in 2-year ARI event. - Disturbed area open for 12-24 months - 1 in 5-year ARI event. - Disturbed are open for >24 months - 1 in 10-year ARI event. - Design capacity excludes minimum 150mm freeboard. - Temporary culvert crossing - minimum 1 in 1-year SRI hydraulic capacity. |
| Erosion control | Erosion control measures | - Minimise exposure of disturbed soils at any time. - Divert water run-off from undisturbed areas around disturbed areas. - Determine the erosion risk rating using local rainfall erosivity, rainfall depth, soil-loss rate or other acceptable methods. - Implement erosion control methods corresponding to identified erosion risk rating. |
| Sediment control | Sediment control measures Design storm for sediment control basins Sediment basin dewatering | - Determine appropriate sediment control measures using: - potential soil loss rate, or - monthly erosivity, or - average monthly rainfall - Collect and drain stormwater from disturbed soils to sediment basin for design storm event: - design storm for sediment basin sizing is 80th% five-day event or similar - Site discharge during sediment basin dewatering: - TSS < 50 mg/L TSS, and - Turbidity not >10% receiving waters turbidity, and - pH 6.5–8.5 |
| Water quality | Litter and other waste, hydrocarbons and other contaminants | - Avoid wind-blown litter; remove gross pollutants. - Ensure there is no visible oil or grease sheen on released waters. - Dispose of waste containing contaminants at authorised facilities. |
| Waterway stability and flood flow management | Changes to the natural waterway hydraulics and hydrology | - For peak flow for the 1-year and 100-year ARI event, use constructed sediment basins to attenuate the discharge rate of stormwater from the site. |