Table 9.2.1.1 Accommodation Activities Code

| Performance Outcomes | Acceptable Outcomes | Proposed Solution Explanation of how the development addresses the Acceptable Outcome and/or Performance Outcome. |
| --- | --- | --- |
| For accepted, accepted subject to requirements and assessable development (code, code (fast tracked) and impact) |  |  |
| Dwelling House (Second Dwelling) Editor’s note—consider Schedule 6, Part 2, Item 2 of the Planning Regulation where in the Low density residential zone. |  |  |
| PO1 A secondary dwelling: - is located on a site with sufficient area to accommodate the dwelling house and the additional dwelling; - maintains an acceptable level of residential amenity for occupants of the additional dwelling, the dwelling house and the adjoining land; and - has a built form that is integrated with the dwelling house. | AO1 A secondary dwelling: - is located on a lot with a minimum site area of 800m²; and - is attached to a garage/carport/breezeway /walkway/patio which is also attached to the dwelling house or alternatively, is located no greater distance from the dwelling house than: - 10 metres, where in the Low density residential zone, Medium density residential zone or Township zone; or - 50 metres, where in the Rural residential zone or Rural zone; and - has a maximum gross floor area of 80m2 or 50% of the gross floor area of the existing dwelling house, whichever is the lesser; and - is located on the same lot as the dwelling house; and - provides a minimum of one (1) additional car parking space. |  |
| Dwelling House (Lots under 450m2) |  |  |
| Site Coverage |  |  |
| PO2 Buildings on lots under 450m2 must include an appropriate balance of building form and open space. | AO2 Site cover is a maximum of 60% of the total site area. |  |
| Building Height |  |  |
| PO3 All buildings must be limited in height to complement the local neighbourhood character and to protect the residential amenity of adjoining lots. | AO3 Buildings and structures have a maximum building height of 8.5 metres and not more than 2 storeys above natural ground level. |  |
| Setbacks |  |  |
| PO4 Dwelling houses on lots under 450m2 must provide for sufficient setbacks from boundaries to: - protect and enhance the residential amenity and streetscape; - ensure separation of habitable rooms and private open space from adjoining dwellings; - limit overshadowing of private open space on adjoining sites; and - variation of front setbacks to the streetscape. | AO4.1 Buildings and structures have a minimum setback of 4.5 metres from the primary road frontage, measured to the wall. Note—except where varied by AO5.2. AO4.2 Eaves, window hoods and decks, porches, patios and upper level covered balconies, including support structures and ancillary structures of open construction have a minimum setback of 3.0 metres from the primary road frontage. Note—open construction includes screening that has openings which make it no more than 50% opaque and does not include walls. AO4.3 For a corner allotment, buildings and structures have a minimum setback of 3.0 metres from the secondary road frontage. AO4.4 Buildings shall be setback from the side and rear boundaries no less than: - metres, measured from the outermost projection of that part of the building, which is 4.5 metres or less above ground level; - 2 metres, measured from the outermost projection of that part of the building which is greater than 4.5 metres but not greater than 7.5 metres above ground level; - 2 metres, plus 0.5 metres for every three metres or part thereof, measured from the outermost projection of that part of the building, which is greater than 7.5 metres above ground level. Note—the abovementioned setbacks may be reduced by the horizontal dimension of eaves, fascias, gutters, downpipes, sunhoods and/or privacy screens which extend beyond the outermost face of the external wall of the building. The setback encroachment must not to exceed 0.6 metres. AO4.5 Built to boundary walls: - the building is built to one side boundary only; - have a maximum height of 3.5 metres; - have a maximum setback of 150mm from the side boundary; - have a maximum length of 8.0 metres - where a built to boundary, the wall is punctuated by a wall setback from the boundary, for a length less than 3.0 metres, this length is to be included in the maximum 8.0 metre length; - the aggregate length of built to boundary walls does not exceed 50% of the length of the boundaries; and - must be a rendered finish. AO4.6 Rain water tanks that are located wholly below ground may be located within the front, side and rear boundary setbacks. |  |
| Built Form |  |  |
| PO5 The building must be oriented to the street to facilitate casual surveillance, provide visual interest and to ensure good quality urban design outcomes. | AO5.1 The building has a door, habitable room window or balcony that faces the primary road frontage and secondary road frontage (where applicable). AO5.2 All dwellings have a visible entry from the primary road frontage. |  |
| PO6 The building is designed and sited to achieve an acceptable level of privacy for the occupants of the dwelling and neighbouring dwellings. | AO6.1 Any habitable room window that directly faces a habitable window of another dwelling has one or more of the following characteristics: - has a sill height of 1.7 metres above floor level; - has fixed obscure glazing in any part of the window below 1.7 metres above floor level; - has the view from the habitable room window screened by a structure not greater than 1.8 metres in height that has openings which make it no more than 50 per cent transparent. AO6.2 Where direct views exist into the private open space of an adjoining dwelling from habitable room windows, balconies, verandahs, terraces, decks and other communal or public areas this view is obscured or screened by privacy screens that have openings which make it no more than 50% transparent. |  |
| PO7 Building design, detailing and finishes must incorporate the articulation of roofs, building footprints and fragmentation of building bulk and appearance to: - add visual interest to the streetscape; - provide differentiation between buildings by means of articulation; and - maximise the amenity of adjoining residences. | AO7 The length of a wall does not exceed 15 metres in one plane, without being offset by a minimum of 1.0 metre of building articulation which could be achieved by either decks, balconies, verandahs and/or other projections. |  |
| PO8 The building is oriented to ensure that garages, bathrooms, toilets and laundries do not dominate the streetscape. | AO8.1 The opening to the garage must not exceed a width of 6.0 metres. AO8.2 Bathroom, laundry and toilet windows do not face the street, unless they are obscured by glass or screened by privacy screens that have openings which make it no less than 50% transparent. |  |
| Services |  |  |
| PO9 Service facilities are: - provided to meet the needs of residents - are sited and designed in an unobtrusive and convenient manner; and - are appropriately screened from public view. | AO9 Service facilities include: - an open air clothes drying facility that is a minimum of 10m2 and located in an external, ventilated and convenient location that is screened from the streetscape or public view; - a waste and recycling bin storage area that is sited more than 6 metres from the road frontage and capable of accommodating two waste bins. |  |
| Private Open Space |  |  |
| PO10 Private open space must have sufficient area to: - suit the recreation needs of residents; - provide for service functions such as clothes drying. | AO10.1 Private open space is provided for each dwelling which comprises: - a minimum area of 25m2 with no part having a minimum dimension of less than 3 metres; - an eastern or northern orientation; and - is directly accessible from a living area. AO10.2 Decks, balconies, verandahs or covered ground level recreation areas such as patios comprise at least 15% of the total private open space area. AO10.3 The slope of the private open space is not more than 1 in 10. |  |
| Landscaping |  |  |
| PO11 Landscaping must be provided to soften the visual effects of the built environment, screen driveways from adjoining properties and add visual interest to the street frontage. | AO11.1 A landscaped area with an average width of 900mm is provided between the driveway and side boundary. AO11.2 Existing street trees are to be retained. AO11.3 A minimum of 30% of the site is to contain functional landscaped open space areas (inclusive of private open space). |  |
| Caretaker's Accommodation |  |  |
| PO12 The provision of Caretaker’s accommodation does not compromise the role, function and operation of the zone. | AO12.1 Caretaker’s accommodation comprises a dwelling with a maximum gross floor area of 100m2. AO12.2 Caretaker’s accommodation must be located where non-residential activities are carried out on the site and the use has a demonstrated need for a caretaker to be on site on a permanent basis. AO12.3 Only one Caretaker’s accommodation is established on a site. |  |
| Multiple dwellings, Dual occupancy, Retirement facility, Residential care facility or Workforce accommodation where for more than one dwelling or accommodation unit and in an Urban Area |  |  |
| Built Form |  |  |
| PO13 The building must be oriented to the street to facilitate casual surveillance, provide visual interest and to ensure good urban design outcomes. | AO13.1 The building has a door, habitable room window or balcony that faces the primary road frontage, and secondary road frontage (where applicable). AO13.2 All dwellings have a visible entry from the primary road frontage. |  |
| PO14 The building is designed and sited to achieve an acceptable level of privacy for the occupants of the dwelling and neighbouring dwellings. | AO14.1 Any habitable room that directly faces a habitable window of another dwelling and has one or more of the following characteristics: - has a sill height of 1.7 metres above floor level; - has fixed obscure glazing in any part of the window below 1.7 metres above floor level; - has the view from the habitable room window screened by a structure not greater than 1.8 metres in height that has openings which make it no more than 50% transparent. AO14.2 Where direct views exist into the private open space of an adjoining dwelling from windows, landing stairs, terraces, decks and other private communal or public areas, this view is obscured or screened by privacy screens that have openings which make it no more than 50% transparent. |  |
| Private Open Space |  |  |
| PO15 Private open space is conveniently located and of a practical size that meets the needs of residents, having regard to: - liveability; - recreation; - privacy; - outdoor entertaining; - landscaping; - amenity; - outlook; and - climate. | AO15.1 Private open space is provided for each dwelling which comprises: - a minimum area of 25m2 with no part having a minimum dimension of less than 3 metres; - has an eastern or northern orientation; and - is directly accessible from a living area. AO15.2 Decks, balconies, verandahs or covered ground level recreation areas such as patios, comprise at least 15% of the total private open space area. Where for multiple dwellings and where dwelling are above ground level AO15.3 Private open space may be provided in the form of a balcony having a minimum area of 8m2 and a minimum dimension of 2 metres and that is directly accessible from a living area. |  |
| Multiple dwellings, Retirement facility, Residential care facility or Workforce accommodation where for more than one dwelling or accommodation unit and in an Urban Area |  |  |
| Landscaping and Communal Open Space |  |  |
| PO16 Landscaped open space contributes to the character and amenity of the site and locality. | AO16.1 A minimum of 15% of the site area is provided as landscaped open space. AO16.2 Acoustic screening is provided adjacent to any vehicle movement or vehicle parking areas along the side or rear boundary. AO16.3 A 1 metre wide vegetated buffer and 1.8 metre screen fence is provided adjacent to any movement or parking areas along the side or rear boundary. |  |
| PO17 Communal open space for recreation is provided where dwellings do not have access to ground level private open space. | AO17 Where dwellings do not have access to ground level private open space, communal open space is provided in accordance with the following: - one area of 50m2; - a minimum dimension of 5 metres; and - must include recreational facilities such as a shaded and landscaped barbecue area. |  |
| Refuse Storage and Collection |  |  |
| PO18 Refuse storage and collection facilities are located in areas that: - provide reasonable standards of amenity for residents; - maintain the amenity of adjoining premises; | AO18.1 Refuse storage is located for convenient use and designed such that it is an outdoor area that is: - no closer than 3 metres to any frontage or dwelling and 1.5 metres to any other site boundary; - enclosed on three sides with a screen wall extending 0.2 metres above the height of the refuse bin storage; or - screened by dense mature planting. AO18.2 Where for 10 or more dwelling units a communal refuse storage area is provided (for a bulk refuse bin) and is serviced by a private contractor. |  |
| Services and Equipment |  |  |
| PO19 Service facilities are provided to meet the needs of residents and are sited and designed in an unobtrusive and convenient manner. | AO19.1 Each dwelling is provided with an open air clothes drying facility that is a minimum of 8m2 and located in an external, ventilated and convenient location that is screened from the streetscape or public view. Note—clothes drying areas are to be provided in addition to private open space or communal open space areas. AO19.2 All equipment ancillary to any buildings or structures are located or screened so as not to be viewed from the road or public open space. Note—equipment does not include solar panels for electricity generation or water heating and does not include antennae and the like. |  |
| Retirement Facility or Residential Care Facility |  |  |
| Mobility and Access |  |  |
| PO20 The use is located on land that has a gradient conducive to aided mobility. | AO20 The following areas are provided with a slope of 1 in 14 or less: - pedestrian movement areas; - private open space; - communal open space; and - communal clothes drying facilities. |  |
| PO21 The pedestrian movement system: - enables residents to easily navigate the site on foot or with the assistance of mobility aids; - provides non-discriminatory access; - incorporates covered or protected walkways, particularly those linking dwelling units with communal facilities; - provides landscaped and comfortable vantage points to rest, socialise and observe surrounding activities; - provides a variety of circulation options; and - links with external pedestrian paths. | AO21 Pedestrian movement areas: - provide continuous access from all dwelling entries to the primary road frontage; - have a minimum width of 2 metres tapered to 3.5 metres when combined with a seating area; - comply with Australian Standard AS1428 —Design for access and mobility; - have a firm, level, well drained non-slip surface; - provide handrails where there are grade changes or other areas of potential risk to pedestrians; - provide a covered principal walkway that links all on-site communal facilities; and - dense landscaping is a minimum of 30% of the site is to contain functional landscaped open space areas (inclusive of private open space). |  |
| Workforce accommodation |  |  |
| Character |  |  |
| PO22 The roof form of workforce accommodation is consistent with the predominant character of roof forms exhibited in the locality. | AO22.1 The workforce accommodation includes one or more of the following roof types with a pitch of 20 degrees or greater: - skillion; - gable; - hipped; - pitched. AO22.2 The workforce accommodation includes eaves with a minimum width of 600mm. |  |
| Landscaping |  |  |
| PO23 Landscaped open space meets the private and communal recreation needs of non-resident workers and contributes to the protection and enhancement of local character. | AO23.1 A minimum of 20% of the front setback area of the premises is landscaped with drought tolerant vegetation with a minimum width of: - 2 metres to the road frontage boundary; and - 1 metre to all side boundaries. AO23.2 Where adjoining a sensitive receptor, a solid fence having a minimum height of 1.8 metres is provided along all side and rear boundaries. |  |
| Rehabilitation |  |  |
| PO24 The agricultural and/or environmental capacity of the site is reinstated and/or enhanced to ensure that the: - sustainable productivity of the land is protected; and - the character and amenity of the site and surrounds is reinstated. | AO24 The site is rehabilitated following cessation of the workforce accommodation use. |  |
| Rural Workers’ Accommodation |  |  |
| PO25 The Rural workers’ accommodation is: - directly associated with an agricultural based rural activity on the same premises; - commensurate with the scale of the primary agricultural operations; and - addressing a demonstrated need for the use to be situated on the premises. | AO25.1 The Rural workers’ accommodation building is limited to the accommodation of: - rural workers employed in rural activities undertaken on the same premises; and - one rural worker for every 100 hectares and up to a maximum of ten rural workers. AO25.2 The agricultural based rural activity is a minimum of 100 hectares in area. |  |
| PO26 Rural workers’ accommodation is provided with amenities commensurate with the needs of the employees and the permanent or seasonal nature of the employment. | AO26.1 The Rural workers’ accommodation is for permanent occupation and is fully self-contained. OR AO26.2 The Rural workers’ accommodation is for seasonal occupation (up to 3 months), is in an approved structure and shares facilities with an existing Dwelling house or Caretaker’s residence. AO26.3 The Rural workers’ accommodation is located within 100 metres of the Dwelling house or Caretaker’s residence. |  |
| Relocatable Home Park and Tourist Parks |  |  |
| PO27 Tourist park accommodation is located: in proximity to a centre zone; or is on a scenic route in an urban area. | AO27 No acceptable outcome. |  |
| Size, Scale and Setbacks |  |  |
| PO28 The use provides suitable levels of buffering, amenity, privacy, and recreation areas commensurate with the reasonable expectations of visitors and residents having regard to the nature of the accommodation use, and the character of the locality. | AO28.1 A Tourist park or Relocatable home park is located on a site with a minimum area of 1 hectare. AO28.2 The site cover for buildings, roofed structures or relocatable homes is a maximum of 40% of the total site area. AO28.3 The development complies with the provision in Table 9.2.1.2 with respect to: - minimum site area for each accommodation type; - setbacks to internal road frontages; - distances to amenities; - distance from refuse storage areas; and - minimum area for communal recreation. AO28.4 The Relocatable home park provides communal recreation facilities for the exclusive use of residents. Facilities include but are not limited to, children's play equipment, swimming pools and barbeque areas. |  |
| PO29 Tourist parks provide a variety of accommodation types to meet the diversity of tourists visiting the Western Downs and in particular promote caravan and recreation vehicle based tourism. | AO29 Tourist parks provide a minimum of 3 caravan/ recreation vehicle sites for every 1 relocatable home or cabin accommodation types. |  |

Table 9.2.1.2—Tourist park and relocatable home requirements

| Aspects | Type of Accommodation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Relocatable home park | Tourist Park |  |  |
|  | Relocatable home | Caravan/RV* | Cabin | Tent |
| Minimum area per site (m²) | 200 | 100 | 150 | 50 |
| Minimum setback from any internal road frontage of a site to the nearest point of any vehicle or structure (m) | 1.5 | 1.5 | 1.5 | N/A |
| Minimum distance to any toilet ablution building on the land (m) | 20 | 20 | 20 | 20 |
| Maximum distance to any amenity building providing toilet, laundry and ablution facility (m) | 100 | 100 | 100 | 100 |
| Minimum distance to any bulk storage refuse bin (m) | 50 | 50 | 50 | 50 |
| Minimum recreation space (percentage of total site area) | 10 per cent |  |  |  |

*Recreation Vehicle.