Table 9.2.5.1 Rural Activities Code

| Performance Outcomes | Acceptable Outcomes | Proposed Solution Explanation of how the development addresses the Acceptable Outcome and/or Performance Outcome. |
| --- | --- | --- |
| For accepted, accepted subject to requirements and assessable development (code, code (fast tracked) and impact) |  |  |
| All Rural Activities |  |  |
| PO1 Administrative areas are integral and subservient to the rural activity. | AO1 Areas for administration purposes do not exceed 10% of the gross floor area of the building or 50m2, whichever is the lesser. |  |
| Rural Industry and Winery |  |  |
| PO2 Sales associated with development are integral and subservient to the rural activity and do not negatively impact on the economic viability of the Western Downs Activity Centre Network. | AO2 Sales associated with the development are undertaken within an area with a maximum gross floor area of 100m². |  |
| Roadside Stall |  |  |
| PO3 The display and sales of goods from rural activities are subservient to the rural activity and do not negatively impact on the economic viability of the Western Downs activity centre network. | AO3 Roadside stalls: - are not located in the road reserve. - have a maximum display area of 10m2. |  |
| Animal Husbandry |  |  |
| Bee keeping |  |  |
| PO4 Bee hives are located a safe distance from sensitive land uses and land intended predominately for accommodating housing and public roads to protect public health and safety. | AO4.1 Bee hives have a minimum setback of 25 metres to any a road frontage. AO4.2 Bee hives have a minimum setback of 200 metres from: - a sensitive land use; or - land in the following zone(s): - residential zone category; - centre zone category; and - township zone. |  |
| Animal Keeping |  |  |
| PO5 Animal keeping: - is undertaken on a site that has a suitable area to provide for adequate setbacks of buildings, pens and waste disposal areas from: - site boundaries; - roads; and - sensitive land uses; - does not cause an adverse impact on the residents of the premises or adjoining sensitive land uses from emissions including, but not limited to: - noise; and - odour. | AO5.1 The site has a minimum area of: - 4,000m2 for a cattery; - 8,000m2 for an aviary or wildlife refuge; - 4,000m2 for every animal stable; - 20,000m2 for a kennel. AO5.2 Buildings and structures for the purposes of Animal Keeping have a minimum setback of: - 20 metres from a road frontage; - 50 metres from any existing dwelling on the same premises; - 50 metres from any existing sensitive land use on adjoining premises; and - 15 metres from any side or rear boundary. |  |
| PO6 The development must be sited, constructed and managed such that: - animals are securely housed; - the generation of any noise does not cause a nuisance to adjoining properties or other sensitive land uses or receptors. | AO6.1 Premises must be fenced to a minimum height of 1.8 metres. Note—the fence must be designed to prevent animals from escaping. AO6.2 Buildings for the accommodation of animals are to be constructed with impervious reinforced concrete floors. AO6.3 Buildings for the accommodation of animals comprise walls that are acoustically treated to limit noise emissions. AO6.4 Animals are kept within roofed buildings or structures at all times and between the hours of 6.00pm and 7.00am. |  |
| PO7 Disposal of solid waste and liquid waste generated by Animal keeping does not result in any on-site or off-site contamination of soil, surface water and ground water, or create any nuisance from odour, dust or vermin. | AO7 No acceptable outcome. |  |
| Aquaculture |  |  |
| PO8 The use is located on a site which has sufficient area to: - accommodate the intensity and scale of the use, including buildings, pens, ponds, other structures and waste disposal areas; and - adequately separating the use from - sensitive land uses. | Where the site has a minimum area of 1 hectare AO8.1 Buildings, structures and areas associated with the use have a minimum setback of: - 10 metres from any road frontage; and - 10 metres from any side or rear boundary. Where the site has a minimum area of 100 hectares AO8.2 Buildings, structures and areas associated with the use have a minimum setback of: - 50 metres from any road frontage; and - 10 metres from any side or rear boundary. |  |
| PO9 The scale of the use: - is consistent with the character and amenity of the zone; - has regard to proximity to sensitive land uses or receptors. | Where the site has a minimum area of 1 hectare AO9.1 The total water surface area does not exceed 50m2. AO9.2 Buildings and structures associated with the use do not exceed a total area of 25m2. Where the site has a minimum area of 100 hectares AO9.3 Buildings and structures associated with the use do not exceed a total area of 100m2. AO9.4 The total water surface area does not exceed 5 hectares. |  |
| PO10 Wastewater effluent and solid waste disposal does not result in an adverse impact on the environment or result in environmental harm or nuisance. | AO10.1 Wastewater is disposed of via: - collection for lawful offsite disposal; or - disposal to the reticulated sewer network. OR Where in the Rural Zone AO10.2 Development is connected to a safe and efficient on-site wastewater disposal system in accordance with Queensland Plumbing and Wastewater Code and Australian Standard AS/NZS3500—Plumbing and drainage. |  |
| Intensive Animal Industry |  |  |
| PO11 Intensive animal industry is located to ensure they do not cause environmental harm or environmental nuisance to sensitive land uses or receptors. | AO11 Operational activities, buildings (other than for accommodation activities or administrative purposes), pens, ponds, structures and waste disposal areas associated with an intensive animal industry comply with minimum setbacks specified in Table 9.2.5.2 – Separation Distances to Residential and Environmentally Sensitive Land Uses. Note—Council recommends that applicants seeking approval for intensive animal industries refer to the relevant industry guidelines (e.g. National guidelines for Beef Cattle Feedlots in Australia) relevant at time of lodgment and that applicants consult with Department of Agriculture and Fisheries prior to the lodgment of a development application. |  |
| PO12 The physical, chemical and biological integrity and quality of the soil is maintained by ensuring nutrient loads do not exceed the buffering capacity of the soil or the landscape. | AO12 No acceptable outcome. |  |
| PO13 Effluent management practices associated with intensive animal industry demonstrates sustainable disposal (to soil and landscape) by ensuring effluent disposal and treatment activities do not cause: - negative impacts on the natural hydrological cycle; - soil, groundwater or surface water salinity; - leaching of nutrients and/or pesticides, into surface water, groundwater or offsite areas that may be at risk (particularly areas down slope). | AO13 No acceptable outcome. |  |
| PO14 The haulage of animals and goods associated with the use does not result in the deterioration of roads used by ensuring: - the roads used as haulage routes are of an adequate standard to accommodate the type and frequency of traffic generated; - haulage routes are maintained including the removal of dirt and other spillage from trucks; and - Haulage routes do not compromise traffic safety or amenity in the area. Note—a road maintenance plan is required in demonstration of compliance with this Performance Outcome. | AO14 No acceptable outcome. |  |

Table 9.2.5.2 – Separation distances to residential and environmentally sensitive land uses

| Use/Activity | Separation Distance (m) |
| --- | --- |
| Poultry Farms | Minimum 1km taken from the closest outside boundary of the shed/conglomeration of sheds. |
| Piggery | Minimum 1.5km taken from the closest outside boundary of the shed/conglomeration of sheds. |
| Feedlots | Minimum 1.5km taken from the outside extremity of the closest animal holding yard. |
| Cattle Dips and Yards | Minimum 200m from the outside extremity of the closest part of the yard or dip. |
| Abattoirs | Minimum 500m from the nearest part of the built facility or effluent disposal area. |
| Dairy Bails and Yards | Minimum 300m from nearest part of the facility. |
| Stock Saleyards | Minimum 500m from the nearest part of the facility used for holding animals. |